In this lab, you will work in the MS-DOS prompt for MPEG encoder part.

The file ntsc.par contains parameters, which the MPEG encoder will use to encode a sequence of video frames.  In addition to the ntsc.par, you will also need the two other files, inter.mat and intra.mat.  These two *.mat files provide quantization matrices.

MPEG2ENC.EXE

This file encodes a sequence of video frames into distinct I, P or B frames in a MPEG sequence.  The encoded sequences (*.Y, *.U, and *.V) are saved in the directory c:\temp.  To encode, you can type the following.

> mpeg2enc ntsc.par c:\temp\out.m2v

After this, you will get all your outputs in the directory.  The c:\temp includes all the video frames and a file called stat.out, which includes the statistics of the coding (for example, bit-rate and SNR).

NTSC.PAR

The parameters that you will likely to change are the following:

- Name of the source file.  You need to change the directory name and the prefix of the file name.

- Number of frames to encode Default setting is 100.

- N (# of frames in GOP)  This is the total number of frames between your I frames. (ex. If you want ibbpbbpbbpbb, N is 12).

- M (I/P frame distance) M for the previous example is 3.

- Bit_rate (bits/s) Original setting is 5,000,000 bits/s = 5000 kbps

RAWVIEW

To view a particular frame and video sequence, use program rawview.   Please follow the guideline below:

- Double click rawview icon to activate a program.

- At the menu, select options and parameters.

- In parameters window, select source type as Y,U,V , source size as CIF, Choose 1st frame – click select and go to c:\temp and select the first y file, for example, mcif000.y and click open.  Then select chroma format at 4:2:0.  Finally, click ok.

- Click play to play YUV video sequence.  Here you have options to select fast forward and reverse. Click on Pause if you want to view a particular frame.

Then you can select which Y picture to be printed out and turned in.

To print out a *.Y picture by using matlab program, you can use the following code:

clear;

fid = fopen('mcif000.y','r');

TT = fread(fid);

% for cif picture

N=288;

M=352;

for i=1:N

for j=1:M

imageout(i,j)=TT((i-1)*M+j);

end

end

imshow(imageout/255)